# TP-001 – Inspection and Maintainability Verification Procedure

| Document ID | TP-001 |
| --- | --- |
| Revision | 1.0 |
| Project | TOMTOM ISR Demonstrator |
| Verification Method | Inspection / Demonstration |
| Verification Level | Component / Assembly |
| Verification Event | VE-001 – Bench Bring-Up |
| Related Test Report | TR-001 |
| Status | Draft |

## 1. Purpose

Verify that the TOMTOM Ground Control Station enclosure and major hardware are correctly assembled, properly identified, mechanically secure, accessible for maintenance, and ready for functional testing.

## 2. Applicable Requirements

STK-008 – Transportable by one person

SYS-MNT-001 through SYS-MNT-006

SYS-PWR-006 – Reverse-polarity protection

Applicable mechanical and GCS interface requirements

## 3. Required Equipment

TOMTOM GCS

Inspection flashlight

Digital camera/phone

Phillips screwdriver set

Hex keys

Digital multimeter

Verification workbook

## 4. Preconditions

GCS assembly complete.

All hardware installed.

Power disconnected.

Latest configuration recorded.

## 5. Safety Precautions

Disconnect all power sources before opening the enclosure. Use ESD precautions when handling electronics. Do not energize the system during this procedure unless specifically required.

## 6. Inspection Procedure

Record enclosure serial/configuration ID.

Inspect exterior for dents, cracks, corrosion, or loose hardware.

Verify handles, hinges, latches, lid stays, and wheels operate correctly.

Open enclosure and inspect internal mounting hardware.

Verify all fasteners are installed and tightened.

Inspect cable routing for abrasion, pinch points, and strain relief.

Verify connectors are labeled and accessible.

Inspect fuse holders and power distribution wiring.

Verify cooling path is unobstructed.

Inspect USB, Ethernet, DC, and RF connectors for damage.

Verify removable components can be accessed without unnecessary disassembly.

Verify battery compartment accessibility.

Inspect laptop/display mounting security.

Photograph overall interior and exterior.

Record discrepancies in verification workbook.

## 7. Data Collection

| Inspection Item | Condition | Pass/Fail | Comments |
| --- | --- | --- | --- |
| Exterior enclosure |  |  |  |
| Hinges/latches |  |  |  |
| Fasteners |  |  |  |
| Cable routing |  |  |  |
| Connector condition |  |  |  |
| Labels |  |  |  |
| Cooling |  |  |  |
| Power wiring |  |  |  |
| Mounting hardware |  |  |  |
| Accessibility |  |  |  |

## 8. Acceptance Criteria

No structural damage affecting operation.

All hardware securely mounted.

No loose wiring or exposed conductors.

Connectors undamaged and accessible.

Required labels present and legible.

No inspection item marked Fail.

## 9. Required Evidence

Exterior photographs

Interior photographs

Annotated discrepancy photographs (if any)

Completed inspection checklist

Verification workbook entry

## 10. Post-Test Actions

Correct identified discrepancies where practical, update the verification workbook, and authorize progression to TP-012 Power System Verification if all acceptance criteria are satisfied.

## 11. Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-07-12 | Serena Green | Initial Release |