# TP-002 – Functional Bench Test Procedure

| Document ID | TP-002 |
| --- | --- |
| Revision | 1.0 |
| Project | TOMTOM ISR Demonstrator |
| Verification Method | Test |
| Verification Level | Subsystem / System |
| Verification Event | VE-001 – Bench Bring-Up |
| Related Test Report | TR-002 |
| Status | Draft |

## 1. Purpose

Verify that the TOMTOM hardware and software successfully power on, initialize, and perform basic functions before integration testing.

## 2. Applicable Requirements

SYS-FUN-001 through SYS-FUN-005

SYS-REL-001 through SYS-REL-005

SYS-PWR-001 through SYS-PWR-004

SYS-GCS-001

## 3. Required Equipment

TOMTOM GCS

Mission computer

Power source

Digital multimeter

USB peripherals

Ethernet cable

Verification workbook

## 4. Preconditions

TP-001 successfully completed.

All hardware installed.

Battery charged or bench supply connected.

Software installed and configured.

## 5. Safety Precautions

Keep propellers removed or disconnected. Verify correct polarity before applying power. Disconnect power before modifying wiring.

## 6. Test Procedure

Record hardware configuration and software versions.

Measure and record input voltage.

Apply power to the GCS.

Verify normal system startup.

Record boot time.

Verify operating system loads without errors.

Launch QGroundControl.

Verify application startup.

Verify ATAK/OpenTAKServer startup (if installed).

Verify CPU temperature is within normal range.

Verify cooling fan operation.

Verify battery status display.

Verify USB devices are detected.

Verify Ethernet interface is operational.

Verify storage device is accessible.

Verify system clock is correct.

Verify event logging is enabled.

Perform orderly shutdown.

Inspect hardware for abnormal heating or odor.

Record all discrepancies.

## 7. Data Collection

| Parameter | Expected | Actual | Pass/Fail |
| --- | --- | --- | --- |
| Input Voltage |  |  |  |
| Boot Time |  |  |  |
| CPU Temp |  |  |  |
| Fan Operation |  |  |  |
| Battery Status |  |  |  |
| USB |  |  |  |
| Ethernet |  |  |  |
| Storage |  |  |  |
| Software Startup |  |  |  |

## 8. Acceptance Criteria

System powers on successfully.

Operating system boots without critical errors.

Required software launches successfully.

No abnormal heating, smoke, or electrical odor.

Required interfaces function correctly.

No critical faults remain at completion.

## 9. Evidence Required

Boot screen photo

Desktop screenshot

QGroundControl screenshot

Voltage measurements

Completed data table

Verification workbook entry

## 10. Post-Test Actions

Update the verification workbook, document anomalies in TR-002, correct failures as required, and proceed to TP-011 only after all critical acceptance criteria are met.

## 11. Revision History

| Rev | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-07-12 | Serena Green | Initial Release |