# TP-003: Ground Integration Test Procedure

| Document ID | TP-003 |
| --- | --- |
| Revision | 1.0 |
| Project | TOMTOM ISR Demonstrator |
| Verification Method | Test / Demonstration |
| Verification Level | System |
| Verification Event | VE-002 – Ground System Integration |
| Related Test Report | TR-003 |
| Status | Draft |

## 1. Purpose

Verify end-to-end integration of the Ground Control Station, communications hardware, mission software, and air vehicle while remaining on the ground. This procedure confirms that all major subsystems operate together before flight testing.

## 2. Applicable Requirements

SYS-GCS-001 through SYS-GCS-006

SYS-INT-001 through SYS-INT-005

SYS-COM-001 through SYS-COM-006

SYS-MIS-001 through SYS-MIS-006

SYS-HMI-001 through SYS-HMI-006

STK-010

## 3. Required Equipment

TOMTOM Ground Control Station

Air vehicle

Flight battery

Mission computer

Telemetry radio(s)

Ethernet cable

USB cables

Digital multimeter

Verification workbook

Camera for documentation

## 4. Preconditions

TP-001 and TP-002 completed successfully.

Propellers removed or otherwise made safe.

Air vehicle firmware configured.

GCS software configured.

All hardware connections verified.

## 5. Safety Precautions

Remove or restrain propellers before energizing the air vehicle. Verify battery polarity before connection. Keep personnel clear of moving components and disconnect power before modifying wiring.

## 6. Test Configuration

Configure the GCS, mission computer, radios, and air vehicle exactly as intended for operational use. Record software versions, firmware versions, serial numbers, and configuration identifiers in the verification workbook before beginning the test.

## 7. Ground Integration Procedure

Record configuration baseline.

Connect the GCS power source.

Power on the Ground Control Station.

Power on the air vehicle.

Verify telemetry radio initialization.

Establish MAVLink communications.

Verify vehicle appears in QGroundControl.

Verify vehicle position displays correctly.

Verify battery status is displayed.

Verify flight mode indication.

Verify command-and-control link status.

Verify live video stream, if available.

Upload a sample mission.

Validate uploaded mission against the planned route.

Command camera movement (if installed).

Verify still-image and video recording commands.

Verify operator alerts and notifications.

Verify Return-to-Launch command is available (do not execute motors).

Disconnect the telemetry link and verify degraded-link indication.

Reconnect the telemetry link and verify recovery.

Perform an orderly shutdown of the air vehicle.

Shut down the Ground Control Station.

Record discrepancies and observations.

## 8. Data Collection

| Verification Item | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- |
| Vehicle Connection |  |  |  |
| Telemetry |  |  |  |
| Mission Upload |  |  |  |
| Map Display |  |  |  |
| Battery Status |  |  |  |
| Flight Mode |  |  |  |
| Video Feed |  |  |  |
| Camera Control |  |  |  |
| Link Recovery |  |  |  |
| System Shutdown |  |  |  |

## 9. Acceptance Criteria

Ground Control Station successfully communicates with the air vehicle.

Mission upload completes without error.

Required telemetry is displayed.

Operator commands are acknowledged.

Any installed payload functions operate correctly.

No critical hardware or software faults remain at completion.

## 10. Required Evidence

Photographs of integrated setup

QGroundControl screenshots

Mission upload confirmation

Telemetry screenshots

Verification workbook entry

Completed TR-003

## 11. Post-Test Actions

Document all anomalies, update the verification workbook, attach supporting evidence, and resolve critical discrepancies before proceeding to TP-004 Communications Test.

## 12. Revision History

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-07-13 | Serena Green | Initial Release |