# TP-004: Communications Test Procedure

| Document ID | TP-004 |
| --- | --- |
| Revision | 1.0 |
| Project | TOMTOM ISR Demonstrator |
| Verification Method | Test |
| Verification Level | System |
| Verification Event | VE-003: Communications Integration |
| Related Test Report | TR-004 |
| Status | Draft |

## 1. Purpose

Verify command and control, telemetry, video, and network communications between the TOMTOM Ground Control Station and the air vehicle before flight testing.

## 2. Applicable Requirements

SYS-COM-001 through SYS-COM-006

SYS-INT-001 through SYS-INT-005

SYS-PER-003

SYS-PER-004

SYS-GCS-001

SYS-GCS-005

## 3. Required Equipment

TOMTOM Ground Control Station

Air vehicle

Telemetry radio(s)

Mission computer

Ethernet cable

USB cables

Digital multimeter

Network utility software

Verification workbook

## 4. Preconditions

TP-001 through TP-003 completed successfully.

Ground integration complete.

Air vehicle configured and powered.

Mission software operational.

## 5. Safety Precautions

Remove or restrain propellers. Maintain separation between personnel and energized equipment. Disconnect power before modifying communication hardware.

## 6. Test Configuration

Configure the GCS, radios, and air vehicle using the intended operational configuration. Record software versions, radio settings, IP addresses, COM ports, and firmware revisions.

## 7. Communications Test Procedure

Record configuration baseline.

Power on the Ground Control Station.

Power on the air vehicle.

Verify telemetry radio initialization.

Verify MAVLink connection.

Verify command-and-control link status.

Verify telemetry updates at the GCS.

Measure and record telemetry update rate.

Verify vehicle position updates on the map.

Verify flight mode updates.

Verify battery status updates.

Verify live video stream, if installed.

Upload a sample mission.

Verify mission upload acknowledgement.

Temporarily interrupt the telemetry link.

Verify operator receives a degraded or lost-link indication.

Restore the communication link.

Verify automatic reconnection attempt and successful recovery.

Verify command response following reconnection.

Record all observations and discrepancies.

Perform an orderly shutdown.

## 8. Data Collection

| Verification Item | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- |
| Telemetry Link |  |  |  |
| MAVLink Connection |  |  |  |
| Mission Upload |  |  |  |
| Telemetry Rate |  |  |  |
| Vehicle Position |  |  |  |
| Flight Mode |  |  |  |
| Battery Status |  |  |  |
| Video Stream |  |  |  |
| Link Recovery |  |  |  |
| Command Response |  |  |  |

## 9. Acceptance Criteria

Stable command-and-control communications established.

Telemetry updates meet the required rate.

Mission upload completes successfully.

Operator receives degraded-link notification.

System successfully reconnects after temporary link interruption.

No critical communication faults remain.

## 10. Required Evidence

QGroundControl screenshots

Telemetry logs

MAVLink log files

Video capture (if applicable)

Verification workbook entry

Completed TR-004

## 11. Post-Test Actions

Update the verification workbook, document anomalies in TR-004, resolve critical communication issues, and obtain approval before proceeding to payload verification.

## 12. Revision History

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-07-13 | Serena Green | Initial Release |