# TP-005: Payload Verification Procedure

| Document ID | TP-005 |
| --- | --- |
| Revision | 1.0 |
| Project | TOMTOM ISR Demonstrator |
| Verification Method | Test / Demonstration |
| Verification Level | Subsystem / System |
| Verification Event | VE-004: Payload Integration |
| Related Test Report | TR-005 |
| Status | Draft |

## 1. Purpose

Verify that the electro-optical payload, camera controls, live video, image capture, video recording, and associated metadata function correctly before flight operations.

## 2. Applicable Requirements

SYS-PAY-001 through SYS-PAY-006

SYS-FUN-020 through SYS-FUN-024

SYS-COM-003

SYS-GCS-005

SYS-NAV-006

STK-003

## 3. Required Equipment

TOMTOM Ground Control Station

Air vehicle with payload installed

Mission battery

Telemetry radio(s)

Video receiver or network connection

Mission computer

Calibration target or test scene

Verification workbook

## 4. Preconditions

TP-001 through TP-004 completed successfully.

Payload installed and secured.

Communications link established.

Video streaming software operational.

## 5. Safety Precautions

Remove or restrain propellers before energizing the air vehicle. Avoid looking directly into high-intensity LEDs or laser devices if installed.

## 6. Test Configuration

Configure the payload using the intended operational settings. Record camera resolution, frame rate, storage location, firmware version, and payload configuration in the verification workbook.

## 7. Payload Verification Procedure

Record payload configuration baseline.

Power on the Ground Control Station.

Power on the air vehicle.

Verify payload initialization.

Verify live video is displayed on the GCS.

Confirm video quality and image stability.

Verify camera orientation controls.

Exercise full camera movement range, if applicable.

Verify digital zoom operation.

Capture a still image.

Verify image is saved successfully.

Start video recording.

Record at least 30 seconds of video.

Stop video recording.

Verify recorded video file is accessible.

Verify captured media includes mission time information.

Verify captured media includes available position information.

Verify payload operational status is displayed.

Record observations and discrepancies.

Perform an orderly shutdown.

## 8. Data Collection

| Verification Item | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- |
| Payload Startup |  |  |  |
| Live Video |  |  |  |
| Camera Controls |  |  |  |
| Digital Zoom |  |  |  |
| Still Image |  |  |  |
| Video Recording |  |  |  |
| Playback |  |  |  |
| Timestamp Metadata |  |  |  |
| Position Metadata |  |  |  |
| Payload Status |  |  |  |

## 9. Acceptance Criteria

Payload initializes without error.

Live video is displayed continuously.

Camera controls respond correctly.

Still images and video recordings are successfully stored.

Recorded media is readable.

Required metadata is associated with captured media.

No critical payload faults remain.

## 10. Required Evidence

Photograph of payload installation

Screenshot of live video

Sample still image

Sample recorded video

Verification workbook entry

Completed TR-005

## 11. Post-Test Actions

Update the verification workbook, document anomalies in TR-005, correct critical payload issues, and obtain approval before proceeding to TP-006: Navigation Verification.

## 12. Revision History

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-07-13 | Serena Green | Initial Release |