# TP-006: Navigation Verification Procedure

| Document ID | TP-006 |
| --- | --- |
| Revision | 1.0 |
| Project | TOMTOM ISR Demonstrator |
| Verification Method | Test / Demonstration |
| Verification Level | Subsystem / System |
| Verification Event | VE-004: Navigation Verification |
| Related Test Report | TR-006 |
| Status | Draft |

## 1. Purpose

Verify that the navigation subsystem provides accurate position, altitude, heading, waypoint navigation, degraded-GNSS behavior, and geotagging support prior to flight testing.

## 2. Applicable Requirements

SYS-NAV-001 through SYS-NAV-006

SYS-FUN-002

OPS-003

STK-005

STK-006

SYS-PER-003

## 3. Required Equipment

TOMTOM Ground Control Station

Air vehicle

GNSS receiver

Telemetry radio

Mission battery

Mission computer

Known reference location

Verification workbook

## 4. Preconditions

TP-001 through TP-005 completed successfully.

Communications established.

Mission software operational.

Navigation sensors calibrated.

## 5. Safety Precautions

Remove or restrain propellers during bench testing. Conduct any powered navigation checks in a safe, open area. Disconnect power before modifying navigation hardware.

## 6. Test Configuration

Record firmware version, GNSS configuration, home position, waypoint file, map source, coordinate system, and sensor calibration status before beginning the procedure.

## 7. Navigation Verification Procedure

Record navigation configuration baseline.

Power on the GCS.

Power on the air vehicle.

Verify GNSS receiver initialization.

Verify home position acquisition.

Verify vehicle position on the map.

Compare displayed position with a known reference location.

Verify altitude display.

Verify heading indication.

Verify telemetry updates for navigation data.

Create a sample waypoint mission.

Upload the waypoint mission.

Verify uploaded waypoints match the planned mission.

Review estimated mission path.

Verify geotagging is enabled for imagery.

Simulate temporary GNSS degradation if supported.

Verify degraded navigation indication is displayed.

Restore normal GNSS operation.

Verify navigation data returns to nominal operation.

Record observations, measurements, and discrepancies.

Perform an orderly shutdown.

## 8. Data Collection

| Verification Item | Expected Result | Actual Result | Pass/Fail |
| --- | --- | --- | --- |
| GNSS Lock |  |  |  |
| Position Accuracy |  |  |  |
| Altitude |  |  |  |
| Heading |  |  |  |
| Waypoint Upload |  |  |  |
| Mission Path |  |  |  |
| Geotagging |  |  |  |
| Navigation Alerts |  |  |  |
| Recovery |  |  |  |
| Shutdown |  |  |  |

## 9. Acceptance Criteria

Navigation solution initializes successfully.

Vehicle position is displayed correctly.

Altitude and heading update correctly.

Waypoint mission uploads without error.

Geotagging functions correctly when enabled.

Navigation warnings are generated during degraded conditions.

No critical navigation faults remain.

## 10. Required Evidence

Map screenshot showing vehicle position

Waypoint mission screenshot

Navigation telemetry log

GNSS status screenshot

Verification workbook entry

Completed TR-006

## 11. Post-Test Actions

Update the verification workbook, document anomalies in TR-006, resolve critical navigation issues, and obtain approval before proceeding to TP-008: Safety and Fault Response Verification.

## 12. Revision History

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-07-13 | Serena Green | Initial Release |