# TP-012: Power System Verification Procedure

| Document ID | TP-012 |
| --- | --- |
| Revision | 1.0 |
| Project | TOMTOM ISR Demonstrator |
| Verification Method | Test / Inspection |
| Verification Level | Subsystem / System |
| Verification Event | VE-011: Power System Verification |
| Related Test Report | TR-012 |
| Status | Draft |

## 1. Purpose

Verify that the TOMTOM power system safely distributes electrical power to all required hardware, accurately monitors battery condition, and performs expected protective functions before operational use.

## 2. Applicable Requirements

SYS-PWR-001 through SYS-PWR-006

SYS-SAF-001

SYS-SAF-004

SYS-GCS-004

SYS-PER-005

## 3. Required Equipment

TOMTOM Ground Control Station

Primary battery

External DC power supply

Digital multimeter

Clamp meter (optional)

Battery charger

Verification workbook

## 4. Preconditions

TP-001 completed successfully.

Power wiring inspected.

Battery fully charged.

Protective fuses installed.

## 5. Safety Precautions

Verify correct polarity before connecting power. Never bypass fuses or protective devices. Disconnect power before modifying electrical wiring.

## 6. Test Configuration

Record battery type, state of charge, nominal voltage, measured input voltage, fuse ratings, DC-DC converter configuration, and connected loads before beginning testing.

## 7. Power System Verification Procedure

Record configuration baseline.

Inspect battery condition.

Verify fuse installation.

Measure battery voltage before connection.

Connect primary power source.

Measure system input voltage.

Verify DC distribution voltage.

Verify regulated output voltages.

Power on the Ground Control Station.

Verify stable startup.

Measure voltage under load.

Verify battery status indication.

Verify charging indication if applicable.

Verify power is supplied to all required subsystems.

Disconnect and reconnect external power if applicable.

Verify system response to power interruption.

Inspect for abnormal heating.

Inspect wiring and connectors.

Perform orderly shutdown.

Disconnect battery.

Record measurements and discrepancies.

## 8. Data Collection

| Measurement | Expected | Actual | Pass/Fail |
| --- | --- | --- | --- |
| Battery Voltage |  |  |  |
| Input Voltage |  |  |  |
| 5 V Output |  |  |  |
| 12 V Output |  |  |  |
| Battery Indicator |  |  |  |
| Current Draw |  |  |  |
| Power Interruption |  |  |  |
| Shutdown |  |  |  |

## 9. Acceptance Criteria

Battery voltage within expected limits.

Regulated outputs remain within specification.

Power distribution operates correctly.

Battery monitoring functions correctly.

No abnormal heating, smoke, or odor.

No critical electrical faults remain.

## 10. Required Evidence

Voltage measurements

Photographs of power system

Multimeter photographs

Verification workbook entry

Completed TR-012

## 11. Post-Test Actions

Complete TR-012, archive measurement data, update the verification workbook, correct any power-related deficiencies, and authorize progression to integrated operational testing.

## 12. Revision History

| Revision | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-07-13 | Serena Green | Initial Release |